of Switzerland

of Switzerland

The Philosophy of Excellence

The

M Series

5.1 Decoder Module

of Switzerland : Dd lxc

Contents

5.1 Decoder Module Installation 5.1 Decoder connections Line outputs Digital inputs 5.1 Decoder menu Room sound programs Sound field description Pure Analogue Amplifier menu Level setting Level resetting Amplifier setup Speaker configuration Adjusting volumes Speaker distances Center distance Surround speaker distance Quit distance Subwoofer Home Cinema glossary Technical data Notes Guarantee

2 3 4 4 5 6 6 7 8 8 9 9 10 11 11 12 13 13 14 14-16 17 18 19 19

5.1 Decoder Module

The 5.1 Decoder module forms the core of the M51’s audio processing. The 5.1 Decoder plug-in card is made up of no less than 358 individual components. It monitors and controls all the audio signals; it generates all the home cinema signals and sends them on to the outputs.

The sound processors of the 5.1 Decoder module can handle all the common audio formats such as Dolby Pro Logic , Dolby Digital, DTS or MPEG, also in 24 bit high resolution of course.

Additionally, these signals are also made available to external devices over 6 output sockets.

In ultra linear stereo operation, the 5.1 Decoder module handles the conversion of symmetrical  analogue signals. From there, they go over gold-plated silver contact relays to the outputs.

The 5.1 Decoder module is also responsible for the decrypting of digital sources. Here, there are two separate digital inputs available.

 Important

The 5.1 Decoder module plays a very significant role for the whole system. Since it represents the Audio Control Centre, the system will not operate without the decoder module.

Installation

The 5.1 Decoder plug-in card has the most modern technology on it and should be handled with care.

Before you start to install the card, the M51must be switched off and the power plug must be removed from the power socket.

Slot 7 is reserved for the Decoder module, right next to the speaker connections. No other module should be plugged in to this slot because this provides the shortest possible signal route to the outputs.

Loosen the two screws with the TORX screwdriver supplied and remove the slot blanking plate.

Before removing the Decoder module from its packaging, make sure that you are not charged with any static electricity. This could cause a damaging power discharge when you touch the module.

To be on the safe side, you should discharge any static from yourself by touching an earthed metal object such as a radiator.

Remove the decoder module from its packaging and insert it into the slot position, using the two tracks within the M51to guide it into position. The lettering on the card must be at the top. Just before the card is fully inserted, you will feel a certain mechanical resistance as the card meets the M51 contact strip. Push the card fully home by pushing in the area where the 2 screw holes are located and fix it in place with the two screws.

All further steps needed to integrate the new module into the M51will be carried out automatically next time the unit is switched on.

5.1 Decoder connections

Line outputs

### The 5.1 Decoder offers 6 line outputs where audio signals are available in 5.1 format. The following is an overview of the individual signals:

- OUTPUT  L
Line output for the left-hand front channel

- OUTPUT  R
Line output for the right-hand front channel

- SUB
Line output for the active subwoofer

- Center
Line output for the centre channel

- Surround  L
Line output for the rear left-hand surround channel

- Surround  R
Line output for the rear right hand surround channel

These line outputs are used to supply external outputs. In the same way, active speakers can be connected. In Home cinema operation, the active subwoofer is principally controlled by the SUB output.

The level of the 6 line outputs is controlled synchronously to the total volume.



Active loudspeakers are speakers with integrated outputs which don't need any amplifier.



The line outputs are not muted during headphone operation.

Digital inputs

The 5.1 Decoder module can transmit digital sources like CDs, MiniDiscs or Digital Audio Tape over the two digital inputs DIGITAL IN 1-2.

The external source should be connected using a coaxial cable to the cinch/ RCA socket on the 5.1 Decoder.

The external digital sources are activated with the two softkeys

in the Source menu.

The externally connected digital sources are not available at the REC OUT output. They also can't be used in neighbouring rooms in Multiroom operation.



As opposed to the analogue music signal, the digital signal can carry more music channels in one cable. This is why only one cable is necessary per device for stereo or multi-channel reproduction with the DIGITAL IN inputs.

5.1 Decoder menu

As the 5.1 Decoder module is the central unit for the processing of audio signals, this module has been assigned its own function key, namely the large key labelled as Amplifier on the front of the M51.



The five large buttons on the front of the M51 are shown as

Setup  Source   Amplifier Power   Disc

in the following chapters.

Room sound programs

The Amplifier button brings you directly into the 5.1 Decoder Main Menu. Here you make your selection from the 6 room sounds:

- Club

- Theater

- X-Stereo

- Surround

- Hall

- Stadium

The individual programmes can be selected by pressing the corresponding softkey and each programme can be set individually for each source.

The inner part of the display shows a graphic of up to 5 speakers, including seating positions. The speakers which are activated by the individual programs are shown dark; the deactivated speakers are shown light.

Sound field description

- Surround
With this setting, the 5.1 decoder always adapts itself to the current audio format of the selected source. With normal stereo sources, the reproduction uses Dolby Pro Logic. 

With DVD reproduction, either the internal M51 DVD player or an external device connected over DIGITAL IN, the 5.1 Audio format which the DVD provides is automatically selected. These are: 
Dolby Digital, DTS, MPEG

In this case, all other room sound programmes are blocked.

- Club
Sound illusion of a small club with a low ceiling and lots of listeners.

- Stadium
Sound illusion of a large stadium with some distance to the stage.

- Theater
Sound illusion of a theatre with a slightly raised box overlooking the stage

- Hall
Sound illusion of a large concert hall with echoes.

- X-Stereo
With this stereo sound, the stereo signal is also sent to the rear surround speakers but as a mirror image. This means that you have stereo reproduction between each pair of speaker.

With a square layout, this can give a fantastic feeling of space.



The 5.1 Decoder notices the sound field of each individual source.



The selected room sound program is shown in the bottom left corner of the display. Here, the symbol DSP indicates that the corresponding room sound is being created with a digital sound processor.

Pure Analogue

Additionally, it is possible to listen to music in pure stereo with out the influence of any room sound program.

In this case, the audio cables are connected directly to the outputs on an ultra linear amplifier path.

All surround speakers are deactivated in this function and are therefore shown as light symbols on the display.

Amplifier menu

You get to the Amplifier sub-menu by pressing the Menu softkey in the Main menu.

The following display appears when the Surround speaker + Center have been selected in the Setup menu :

If only the two front speakers are active, the following display is shown:



The level controller have no function with the Pure Analog sound field.

Level setting

Using the softkeys

these speakers can be brought to the right balance with the main speakers.

This function enables you to customise the sound temporarily to a particular film. Once all settings have been made, the Menu softkey returns you to the Amplifier Main menu.

These settings are automatically reset as soon as another source is selected.

Please note

With Dolby reproduction, it is not possible to raise the subwoofer level above the 0dB range for license reasons.



The base setting for the volume of all speakers is made through the Amplifier Setup menu ( next section).

Reset level

With the Reset softkey, you can reset the previously set level to 0.

Amplifier Setup

Using the Setup menu, you can adapt your loudspeakers to the optimum acoustics with the M51.

This menu is called by pressing and holding the Setup key for some 2 seconds and then pressing the Amplifier key in the display that follows.

Now you are in the 5.1 Decoder Setup menu.

The graphic in the middle of the display mirrors your listening space.

In the middle is the seating area so the speakers are distributed as follows:

Main speaker, front left

Centre loudspeaker

Main speaker, front right

Right-hand surround speaker

Left-hand surround speaker

The sixth speaker in a 5.1 system, the subwoofer, is not shown in this graphic.

Speaker configuration

Center speaker

Using the Center softkey, you can switch the centre speaker on and off.

If you don't have a centre speaker, this should also be defined through the setup or otherwise some dialog scenes will be lost with Dolby reproduction. If the centre speaker is switched off, the centre signals are reproduced through the front main speakers.

The status of the centre speaker is shown in the display with the  symbol.

Surround speakers

Using the Surround softkey, you can switch the centre speaker on and off.

The status of the surround speakers is shown in the display with the  and  symbols.

Adjusting volumes

Setting up the base volumes for the individual speakers.

As soon as the Noise key is pressed, the M51 produces a noise on the front left speaker for some 3 seconds. During this time, you can set up the volume of the noise using the rotary control knob.

Once this time has elapsed, the 5.1 Decoder starts the adjusting process itself. In this case, the sounds are moved in a clockwise direction around the listener.

This signal can be heard at each speaker for about 2 seconds. During this time, you can set the optimal volume in the range ± 10 dB using the rotary control knob or the remote control.

The goal

The aim of this exercise is that the sound should be experienced at the same intensity from each speaker. This guarantees that effects, dialogue or music reaches the ear in the correct relationship.

This procedure continues until you press the Noise softkey again.

Speaker distances

Distance selection between the listening position and the individual speakers.

The M51 offers the option to make sure that the acoustic signal reaches you at the right time at the listening position. Here, the M51 needs to know the exact distance from the speaker to the listener.

Main speaker distance

The first time you press the Distance softkey, you can set the distance to the front speakers  and  using the rotary control knob. These two speakers should be the same distance from the listener and you can select in the range from 2.0 to 9.9 metres.



1m ≈  40 inch

1m ≈  3.3 feet

Center distance

You can setup the centre speaker distance by pressing the Distance softkey again. Here, you also use the rotary control knob to set the distance.



In order to achieve optimum acoustic reproduction, there should be a certain relationship between the front speaker distances and the centre speaker distance. For this reason, the M51 offers you exactly the correct range to make your selection from.

Example:

Selected distance:

and  = 4.5 m

Optimum distance:

= 3.0 – 4.5 m

If you can't position the centre speaker within this distance range, just choose the nearest alternative distance available to you.

Surround speaker dist.

As opposed to the main speakers, the rear surround speakers can be set at different distances from the listener.

By pressing the Distance softkey one more time, you can set up first the right and then the left rear surround speaker. Also here, you use the rotary control knob to set the distance.



In the same way as with the centre speaker, the M51 only allows a certain distance range in relation to the main speakers

Example:

Selected distance:

and  = 4.5 m

Optimum distance:

and  = 0.0 – 4.5 m

If you can't position the surround speakers within this distance range, just choose the nearest alternative distance available to you.

Quit Distance

Simply press one of the other softkeys in the Setup menu  if you want to quit the Distance menu field.

If you don't want to make any more settings through the

Setup menu, press either the Source or  Amplifier keys.

Subwoofer

Using the Subwoofer menu, you can setup the operation of an active subwoofer in the whole speaker concept.

By repeatedly pressing the Subwoofer key, you will reach the menus shown where you can make the appropriate settings for your use.

Subwoofer

Sur. and Stereo

This mode is recommended if the active subwoofer should work with multi-channel reproduction (Hall, Dolby Pro Logic, etc.) and in Pure Stereo operation.

Such a setting is particularly beneficial if the other speakers can produce bass tones, but only within a limited range. Main compact or shelf-mounted speakers with limited bass reproduction belong to this category.

Subwoofer

Satellite Systems

If you have a system known as a satellite system, e.g.. REVOX Magic Cubes + Magic Bass Active, 5 small speakers in combination with the corresponding active bass, the Satellite Systems  settings is the correct choice.

This will give you increased dynamics and a clearer sound picture.

Subwoofer

None

This mode should be selected if no subwoofer is available.

In this case, the main speakers must be capable of reproducing the deepest bass sounds. This is particularly necessary in the action scenes of DVD films in order to achieve the maximum live atmosphere.

Nonetheless, there is still an unfiltered signal at SUB Out.

Subwoofer

Surround only

In this mode, the subwoofer is only activated if a surround sound such as Theater or Church is selected. In Pure Analog operation, the subwoofer remains silent.

This setting is ideal for the purist listener who likes to experience films and concerts on DVD with all six channels, including the subwoofer, but is happy to rely on just the main speakers for normal stereo operation such as CD or Phono. This assumes, of course, that both main speakers can produce the required deep base tones cleanly.

Quit

To quit the Subwoofer menu, simply press the Menu softkey.

Home Cinema glossary

The world of Home Cinema is full of abbreviations and we have included a few of the more important ones here:

AC-3

Abbreviation for Audio Coding No. 3. To start with, this was a widely used name for Dolby Digital.

Dolby Surround

Established in the cinema since the 70s and in Home Cinema since the 90s as Dolby Stereo, it works with analogue sound as well as earlier produced analogue. It is now replaced with Digital Signal Processors.

Dolby Digital

Current standard in the cinema and the most widely used sound on DVD. In this case with two channels (stereo) or 5 tracks plus subwoofer (5.1 format).

DTS

Competition ( Digital Theater Systems) to Dolby Digital in the cinema as well as on DVD.

DVD Video

12 cm data carriers, similar to a CD, stores on a maximum 18 Gb complete films in more languages and sound formats.

Frequently extended with bonus material, e.g. "The Making of…“.

Country code

Is found on DVDs and DVD players. Both must be the same. Code 2 is planned for the European market. Code 1 DVDs from the USA will not play on such players.

LFE

Low Frequency Effects

The sixth sound track on DVDs with low frequency sound effects.

THX

Technical definition from Lucasfilm as a quality norm in cinemas and living rooms. The goal is that films should always be like they are in the director and preview cinemas in Hollywood.

Widescreen

16:9 picture format or wider. In comparison, the original TV format is 4:3.

Technical data

| Formats: |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dolby Prologic, Dolby Digital (AC3), dts |  |  |  |  |  |  |  |  |  |  |
| MPEG 1 Layer 2, MPEG 2 with/ without extension |  |  |  |  |  |  |  |  |  |  |
| Inputs: |  |  |  |  |  |  |  |  |  |  |
| Digital In 1 | SPDIFF |  | 32, 44.1, 48 kHz |  |  |  |  |  |  |  |
| Digital In 2 | SPDIFF |  | 32, 44.1, 48 kHz |  |  |  |  |  |  |  |
| Analogue outputs: |  |  |  |  |  |  |  |  |  |  |
| Front |  | L / R |  | 2.0 V / 200 Ohm |  | (based on 0dBFS) |  |  |  |  |
| Sub |  |  |  | 2.0 V / 200 Ohm |  | (based on 0dBFS) |  |  |  |  |
| Centre |  |  |  | 2.0 V / 200 Ohm |  | (based on 0dBFS) |  |  |  |  |
| Rear |  | L / R |  | 2.0 V / 200 Ohm |  | (based on 0dBFS) |  |  |  |  |
| Harmonic distortion: 0.005% |  |  |  |  |  |  |  |  |  |  |
| D / A conversion |  |  |  | 6 x 24 bit |  |  |  |  |  |  |
| Low noise spacing : 92dB CCIR-ARM |  |  |  |  |  |  |  |  |  |  |
| Operating conditions |  |  | +10°...40 °C / 50°…104°F (DIN 40040) |  |  |  |  |  |  |  |

Errors excepted

Product subject to modification

Notes

Manufactured under license from Dolby Laboratories. „Dolby“, „Pro-Logic“. „ Dolby Digital “ and the double D-Symbol are trademarks of Dolby Laboratories. Confidential Unpublished Works c 1992-1997 Dolby Laboratories, Inc. All rights reserved.

Manufactured under license from Digital Theatre Systems, Inc. US Pat No. 5,451,942 and other worldwide patents issues and pending. „dts“, „ dts Digital Surround „ are trademarks of Digital Theatre Systems, Inc. C 1996 Digital Theatre Systems, Inc. All rights reserved.

Dolby, Pro-Logic, AC-3 and Dolby Digital are registered trademarks Dolby Laboratories Licensing Corp.

Guarantee

The guarantee period is 24 months from the date of purchase.

Your dealer should be your first contact if you need service.

If he can't give you the help you need, send Decoder module carriage free and without any accessories to your national Sales Office.

Please supply a complete description of the problem and a full return postal address.