of Switzerland

of Switzerland

The Philosophy of Excellence

The

M Series

DVD Module

of Switzerland : Dd lxc

Contents

DVD Module Installation DVD connections SCART connection Digital outputs DVD CD DVD menu DVD/CD operation Search Repeat Repeat CD Repeat DVD Shuffle Stop CD programming Delete programme title Delete complete programme DVD screen menu Setup menu in OSD Menu field Menu field Menu field Menu field Select video format Video mode Home cinema glossary Technical data Notes Guarantee

2 3 4 4 5 6 7 8 9 9 10 11 12 12 13 14 14 15 16 17 18 19 20 21 22 23 24 25 25

DVD module

The DVD Module from REVOX is the audio-visual interface between the DVD drive, video components and the M51.

Features

The high quality and robust DVD drive transfers the digital data to the DVD module. Here, with 24 bit precision, the data is converted to an analogue signal and sent directly over the ultra linear path to the outputs. Alternatively it transfers the data as a digital signal to the decoder module, which handles the conversion into a 5.1 multi-channel format.

Further, the DVD plug-in card makes the connection to the video components using a high quality, switched SCART connector which delivers all commonly used picture formats. Using a coaxial digital output, the audio signal is made available for an external decoder or for digital recording devices such as Minidisc and DAT recorders. In addition, you can connect the REVOX digital speaker system here.

 Important

The 5.1 DVD module plays a very significant role for the whole system. Since it represents the audio-visual interface, the integrated DVD player will not operate without the DVD card.

Installation

The DVD plug-in card has the most modern technology on it and should be handled with care.

Before you start to install the card, the M51must be switched off and the power plug must be removed from the power socket.

Slot 3 is reserved for the DVD module, the third slot from the right viewed from the back of the unit. No other module should be plugged in to this slot because this provides the shortest possible signal route to the outputs.

Loosen the two screws with the TORX screwdriver supplied and remove the slot blanking plate.

Before removing the DVD module from its packaging, make sure that you are not charged with any static electricity. This could cause a damaging power discharge when you touch the module.

To be on the safe side, you should discharge any static from yourself by touching an earthed metal object such as a radiator.

Remove the DVD module from its packaging and insert it into the slot position, using the two tracks within the M51to guide it into position. The lettering on the card must be at the top. Just before the card is fully inserted, you will feel a certain mechanical resistance as the card meets the M51contact strip. Push the card fully home by pushing in the area where the 2 screw holes are located and fix it in place with the two screws.

All further steps needed to integrate the new module into the M51will be carried out automatically next time the unit is switched on.

DVD connections



A special feature of the M51SCART cable from

REVOX is the high quality individual screening of each core which guarantee the optimum transmission performance.

SCART connection

All commonly used picture and sound formats can be handled using the SCART socket (AV Link). The connections of this socket adhere to the international SCART standard.

Using the SCART cable supplied, you can connect the DVD module to the television or video recorder.

Of course, you can also connect a plasma screen or video projector to the SCART socket. If these devices don't have a SCART input, you should purchase the appropriate cable or adapter from your nearest dealer.

The following connections can be made :

SCART – RGB, Video projector

SCART – Y/C, S-VHS

SCART – YUV

SCART – Video Signal

SCART – Audio L/R +

SCART – Audio L/R + S-VHS

Digital output

By connecting a normal cinch cable to the coaxial digital output, the audio signal is made available for an external decoder or for digital recording devices such as Minidisc and DAT recorders. In addition, you can connect the REVOX digital speaker system here.

With normal audio CDs, there is a digital stereo signal (SPDIFF). With DVD reproduction, there is a 5.1 signal at the digital output which is only suitable for a decoder.



As opposed to the analogue music signal, the digital signal can carry more music channels in one cable. This is why only one cable is necessary per device for stereo or multi-channel reproduction with the DIGITAL OUT inputs.

DVD-CD

Both DVDs and CDs are controlled through the DVD menu. Both devices can be played on the M51DVD drive. You can play CD-Rs and CD-RWs as well.



CD-R = CD Recordable

A CD which can be written to once

CD-RW = CD Rewritable

A CD which can be written to more than once

Inserting the disc

The CD drawer is opened by pressing the Disc button on the front of the M51. Insert the CD/DVD, with the reflective side down taking care to center it in the circular recess in the drawer. You can play 8cm mini CDs as well as the standard 12cm CDs.

The CD drawer can be closed by pressing the Disc button again.

Warning

The CD drawer should not have any mechanical load applied to it. You should avoid applying any pressure to the drawer, in order to avoid causing any damage.



The five large buttons on the front of the M51are shown as

Setup  Source   Amplifier Power   Disc

in the following chapters.

DVD Menu

If you have already opened the DVD menu, the disc will start to play as soon as the CD drawer is closed.

If you have not yet opened the DVD menu, you can get to it as follows:

The DVD/CD player is accessed through the Source menu. Press the Source button on the M51and then press the DVD/CD softkey.

Once the DVD/CD menu is opened, depending on the CD or DVD currently inserted, the following menu field appears in the display.

CD Display

The Track number and the current playing time are displayed. The top line also shows whether a CD or a DVD is currently inserted. In this case it shows the text CD.

DVD Display

The Chapter number and the current playing time are displayed. Additionally with DVDs, the Title is also displayed.

The top line also shows whether a CD or a DVD is currently inserted. In this case it shows the text DVD.

If no disc is inserted, the following display appears:

DVD/CD operation

Several commands are available to give you quick and easy operation of your CDs and/or DVDs.

Play

The DVD/CD is started with the Play softkey.

Pause

If you want to pause the disc for a while, simply press the Pause softkey. To cancel the Pause function, press the Play softkey once more and the disc will restart from where it was paused.

Change track

Using the two "previous" and "next" Track softkeys, you can jump to the start of the previous or next Track on a CD or Chapter on a DVD.

Example

You are currently listening to Track 4 on a CD and you want to jump to the start of Track 3. In this case you need to press the "previous" Track softkey twice. The first press brings you to the start of Track 4 and the second press brings you to the start of Track 3.

Search

If you want to search for a particular position within a Track or Chapter, you can use the Search function. This function has two search speeds.

The first time you press the Search softkey, the play speed is accelerated in a single step. The sound continues so that you can listen during the search.

If you press the Search key again, the search speed is increased and in this case the sound is switched off.

Once you have reached the required position, simply press the Play key and the disc will return to normal speed.

Repeat

The Repeat function enables you to repeat and individual Track or the whole CD.

With DVDs, you can choose between three repeat options, Chapter, Title or the complete DVD.

The Repeat function is found on the second page of the CD/DVD menu. The following is an example from the CD menu:

Repeat CD

The various CD Repeat functions can be selected by repeated pressing of the Repeat softkey. This takes you through a loop which is ended when the Repeat function is deactivated.

Repeat Track

Repeat Disc

Deactivate Repeat

If you press the Repeat softkey once, this triggers the repeat of an individual Track. This is shown in the display with the text

Repeat Track

If you activate Repeat while the disc is stopped, this causes the first track to be repeated when you start the CD with Play.

If you press Repeat while a track is playing, the track will be repeated until either Stop is pressed or until the Repeat function is deactivated.

If you press Repeat again, the whole CD will be repeated once. Then the CD stops. This is shown in the display with the text:

Repeat Chap.

Repeat Title

Repeat Disc

[Repeat deaktiviert]

Repeat Disc

A further press on the Repeat softkey ends the repeat function.

The following graphic shows the two repeat functions with a CD.

Repeat DVD

As with the CD, the three DVD Repeat functions can be selected by repeated pressing of the Repeat softkey. This takes you through a loop which is ended when the Repeat function is deactivated.

Repeat Chap.

Repeat Title

Repeat Disc

Deactivate Repeat

If you press the Repeat softkey once, this triggers the repeat of an individual Chapter. This is shown in the display with the text

Repeat Chap.

If you want to repeat a whole Title, press Repeat once more. This is shown in the display with the text

Repeat Title

To repeat a complete DVD, press the Repeat softkey once more. This is shown in the display with the text

Repeat Disc

A further press on the Repeat softkey ends the repeat function.

The following graphic shows the three repeat functions with a DVD.

Shuffle

With the Shuffle function, you can play your discs in a random order. The Shuffle command is on the second page of the CD/DVD menu.

Shuffle can be started in Stop- or Play mode. In both cases, the random selection is started immediately.

If you want to search for a further title in a random order while Shuffle mode is activated, press the "next" Track softkey  from the first page of the CD/DVD menu.

Stop

The Stop softkey stops all actions in the DVD menu and stops the disc currently being played.

In the case of a CD, the following display in Stop mode shows the total playing time of the CD and the number of tracks.

With DVDs, the Stop mode information is limited to showing the number of Titles.

CD programming

Using your TV screen, you can group specified tracks within a CD together into a programme. This can only be seen if your television or Video projector is turned on.

Press the Stop softkey in the CD menu so that the disc is not playing. Now, the following display appears on your TV screen.

The most important information about the CD currently inserted is shown in the top, dark line of the display.

The second line, with the abbreviation fts shows with On  Off if a CD programme has been created. Using the ▼▲ button on your remote control, you can jump to this line. If a program has been created, it can be activated or deactivated by selecting On or Off with the ◄► button on your remote control

The selected setting is shown in blue.

Creating a CD programme

The actual selection of individual CD tracks is done through the next line track. Using the ▼▲ button on your remote control, you can jump to this line where you select the tracks in the required order using the ◄► button and confirm each of your inputs with the OK button on your remote control.

Each track which has been selected and confirmed appears in the program line on the TV screen in the order selected.

This example shows that the Tracks 5 8 2 have been selected in that order. If you now press the Play softkey, only these three Tracks will be played.

Deleting programme tracks

If you want to delete individual titles out of a programme, jump to the program line using the  ▼▲ button on your remote control.

Here, you can select the titles to be deleted with the ◄► button and the selected titles are shown in blue. To delete the titles, press the OK button on your remote.

Delete complete programme

If you want to delete a complete programme, jump to the Clear all line using the  ▼▲ button on your remote control.

Then, to delete the programme, press the OK button on your remote.



The selected program is used as the basis for all other commands within the CD menu such as Track, Search, Repeat or Shuffle.

The CD player reacts as if there were no other tracks available on the CD, other than those selected for the program.

Any programming of a CD is maintained until it is deleted, even if it is removed from the M51 in the meantime.

The M51 can store the programming of up to 100 individual CDs.

DVD screen menu

With DVDs, there are further setup possibilities, in addition to the normal DVD menu.

These functions can be activated through the On-Screen-Display  (OSD).

The upper menu appears at the top edge of the TV or projector screen.

You can select and modify the individual functions using the ◄► , ▼▲ and OK buttons.

The individual options are:

- Title display

- Chapter display

- Language selection

- Sub-title selection

- Viewing angle

- Zoom

- Programme

- Stereo/ Surround

- Variable slow motion

- Slow motion

3

1

4

5

2

6

Fast forward/back

8

9

7

10

11

12

Go to timepoint

Setup menu in OSD

As well as the functions described in the previous chapter, the Setup menu offers you a range of setup options through the OSD menu, which you can operate through your remote.

You should be aware that the settings you make here are not just valid for the disc currently in the DVD but for all DVDs. If a selected setting, e.g. Language English, is not available on the DVD, a preset value will be selected instead.

In order address up the Setup menu, place the positioning arrow in the upper menu bar, on the extreme left-hand side, using the ◄ button on your remote (see the magnifying glass graphic below).

Use the ▼ button, to access the four menu fields which are represented by the four following symbols.

The menus are shown in English. If you want to select a different language, this is done through the Menü option in the       menu.

Menu field

TV Shape

Here, you can select from the various display formats. If different picture formats are stored on the DVD, the preset picture format will be used as the preferred format. You can choose between the 16:9 Cinema format or the 4:3 TV format. The 4:3 format offers two further options: 4:3 Letterbox gives the whole picture contents with black lines top and bottom and 4:3 Panscan which narrows the picture left and right while maintaining the full height.

Black level shift

On activates the optimum density setting; Off deactivates it.

The result is a modification of the colour dynamics and an improvement in the contrast. This is only possible with NSTC applications.

Video shift

If the picture is set too close to the left or right edge of the screen, you can correct this here.

Menu field

Digital output

You must set the mode to All to ensure correct playback.

Analogue output

This setting is not supported.

Night mode

This setting is not supported.

Karaoke vocal

This setting is not supported.

Menu field

Audio language

The soundtrack will be played in the language selected here, if it is available on the DVD. If not, the first language as defined on the DVD will be used for the soundtrack.

Subtitles

The subtitles will be displayed in the language selected here, if it is available on the DVD. If not, the first language as defined on the DVD will be used for the subtitles.

Menu

The OSD menus appear in the language selected here.

# Access control

This allows you to block the replaying of certain DVDs (child lock). You can also ensure that any settings you have made cannot be modified.

# Status window

Switch the display of the Status window in the OSD display on and off.

Menu field

# Low power standby

This setting is not supported as the M51controls it itself.

PBC

PBC is the Play Back Control, which starts the factory-set interactive menus which are on the DVDs. Without PBC, the tracks would be played in the order they occur on the DVD.

Select video format

The M51 DVD module can generate various video formats for different picture playback media.

Depending on whether you have a TV with a CRT, a video projector or a plasma TV, certain video formats will give you the best picture quality.

With the Video Mode function, the DVD module supplies four different formats to the SCART socket. You address the function from the DVD menu by pressing and holding the Setup button on the front of the M51 for approx. 2 seconds and then selecting the Source softkey.

One of the following displays appears:

You can scroll through the various menus by repeated pressing of Video Mode. Select the appropriate setting for your application.

Video Mode:	RGB+CVBS

As the + character in the name implies, this mode contains two signals which are sent simultaneously.

1. RGB
This is the standard signal for many video projectors (beamers) and TV appliances. Here, the video signal is represented by the three primary colours red (R), green (G) and blue (B). These three colour signals can either be transmitted using the REVOX -SCART cable or a special SCART/RGB cable.

2. CVBS 
English name "Composite Video Burst Sync". This name represents the normal video signal. If your TV only has an aerial socket, you can use this video signal, with the appropriate adapter.

Video Mode:	Y/C

This video format is also known as S-VHS. It refers to an analogue video signal which processes the brightness factor (Y) luminance and the colour factor (C) chrominance independently of each other. This enables a higher resolution and colour constancy. These two signals can either be transmitted using the REVOX -SCART cable, a special SCART S-VHS cable or a SCART-hosiden adapter.

Video Mode:	Y/U/V

The abbreviation Y/U/V represents an analogue video signal which handles three independent signal elements, physically separated from each other; one signal (Y) for the brightness and two for the colour elements (U,V).

Many plasma televisions and screens use these signal formats because they come very close to achieving the unrivalled RGB picture quality, but they only need a smaller transfer rate.

These three picture signals can either be transmitted using the REVOX -SCART cable or a special SCART Y/U/V cable.

Home Cinema glossary

The world of Home Cinema is full of abbreviations and we have included a few of the more important ones here:

AC-3

Abbreviation for Audio Coding No. 3. To start with, this was a widely used name for Dolby Digital.

Dolby Surround

Established in the cinema since the 70s and in Home Cinema since the 90s as Dolby Stereo. Works with analogue sound. Earlier produced analogue as well. Now replaced with Digital Signal Processors.

Dolby Digital

Current standard in the cinema and the most widely used sound on DVD. In this case with two channels (stereo) or 5 tracks plus subwoofer (5.1 format).

DTS

Competition ( Digital Theater Systems) to Dolby Digital in the cinema as well as on DVD.

DVD Video

12 cm data carriers, similar to a CD, stores on a maximum 18 Gb complete films in more languages and sound formats.

Frequently extended with bonus material, e.g. "The Making of…“.

Country code

Is found on DVDs and DVD players. Both must be the same. Code 2 is planned for the European market. Code 1 DVDs from the USA will not play on such players.

LFE

Low Frequency Effects

The sixth sound track on DVDs with low frequency sound effects.

THX

Technical definition from Lucasfilm as a quality norm in cinemas and living rooms. The goal is that films should always be like they are in the director and preview cinemas in Hollywood.

Widescreen

16:9 picture format or wider. In comparison, the original TV format is 4:3.

Technical data

| Usable formats |  |  |  |
| --- | --- | --- | --- |
| DVD Video (Single and Dual Layer) |  |  |  |
| Video CD |  |  |  |
| Audio CD ( incl. CD-R and CD-RW) |  |  |  |
| TV Standards: |  |  |  |
| Multi-standard |  | PAL (Europe) | 50Hz/625 lines |
|  |  | NTSC (USA) | 60Hz/525 lines |
| Video signals: |  |  | Plug connection |
| Configurable in SETUP |  |  |  |
| CVBS |  | 1 Vpp at 75 Ohm | SCART |
| S-Video | Y: | 1 Vpp at 75 Ohm | SCART |
|  | C: | 0.3 Vpp at 75 Ohm | SCART |
| YUV | Y: | 1 Vpp at 75 Ohm | SCART |
|  | U: | 0.7 Vpp at 75 Ohm | SCART |
|  | V: | 1 Vpp at 75 Ohm | SCART |
| Switching voltage |  | 12 V, with DVD operation | SCART-PIN 8 |
| Digital Out: |  |  |  |
| PCM 16, 20, 24 Bit |  | SPDIFF up to 48kHz, 75 Cinch |  |

Errors excepted

Product subject to modification

Notes

Manufactured under license from Dolby Laboratories. „Dolby“, „Pro-Logic“. „ Dolby Digital “ and the double D-Symbol are trademarks of Dolby Laboratories. Confidential Unpublished Works c 1992-1997 Dolby Laboratories, Inc. All rights reserved.

Manufactured under license from Digital Theatre Systems, Inc. US Pat No. 5,451,942 and other worldwide patents issues and pending. „dts“, „ dts Digital Surround „ are trademarks of Digital Theatre Systems, Inc. C 1996 Digital Theatre Systems, Inc. All rights reserved.

Dolby, Pro-Logic, AC-3 and Dolby Digital are registered trademarks of the Dolby Laboratories Licensing Corp.

Guarantee

The guarantee period is 24 months from the date of purchase.

Your dealer should be your first contact if you need service.

If he can't give you the help you need, send the DVD Module carriage free and without any accessories to your national Sales Office.

Please supply a complete description of the problem and a full return postal address.